Search Control Panel

s

Open The Network and Sharing Center

Open Windows Defender Firewall

Click Turn Windows Defender Firewall on or off

Turn OFF Windows Firewall during network setup to avoid problem on networking

Open Change Advance Sharing Settings

Turn on network discovery

Turn on file and printing sharing

Turn on sharing so anyone with network access can read and           write files folder

Open Change Adapter Settings

Right Click Ethernet then properties

Click Internet Protocol version 4 (TCP/IPv4) then properties

IP Address must be Obtain

Open browser default IP add 192.168.1.1

User name : smartbro

Password : smartbro

Can change IP Address depends on your server #

192.168.1.10

Subnet mask : 255.255.255.0 and enable

Then save

Set your own SSID / wireless network name

Set Password WPA2-PSK

Set your own password

Then save

DHCP Setting depends on range

Start IP Address : 192.168.1.10

End IP Address : 192.168.1.20

Default Gateway : 192.168.1.1

Then save

DHCP Address Reservationa for IP Addressing

Add MAC Address

To Get MAC Address

CMD : getmac and copy MAC add

MAC Address : E4-A8-DF-B4-14-CA

Reserve IP Address : 192.168.1.11

Then enable and save

Add all MAC address of Server PC, AP,Laptop and Client and deside IP add reservations

Then Reboot to take effect on changes

CMD : Type ipconfig then enter to see physical IP Address

Default Gateway : 192.168.1.1

Type : ipconfig/release

Then ipconfig/renew to configure your IP address

- IPv4………………………. : 192.168.1.18

- Subnet Mask ………. : 255.255.255.0

- Default Gateway…….. : 192.168.1.1

IP Address Reservation List

Server PC – 192.168.1.11

Client PC – 192.168.1.12

Laptop  – 192.168.1.13

AP – 192.168.1.14

Router  – 192.168.1.1

NOTE: Acesas Point (AP) same configuration on router and AP must be STATIC setup then connect to Laptop via Wifi

Then Ping all IP Add if connected to loop network topology